The Ring-Centered Waveguide Flange for

Submillimeter Wavelengths

A. R. Kerr and S. Srikanth

National Radio Astronomy Observatory, Charlottesville, VA 22903

Contact:  akerr@nrao.edu, phone +1-434-296-0363

Abstract—The standard UG-387 waveguide flange is not well suited for use above ~110 GHz.  The MIL Spec tolerances allow a misalignment between waveguides as large as 0.006", which results in a 23 dB return loss at the upper end of the WR-10 band (75-110 GHz) [1].  Modified versions of the UG-387 flange, with tighter tolerances and smaller clearance between pins and pin holes, are in use for bands as small as WR-3 and beyond [1-3], but the practical limit of waveguide alignment using flanges with non-interfering pins appears to be about 0.0028", which gives a minimum return loss between waveguides of 23 dB in the WR-3 band (220-330 GHz).  This degree of misalignment is unacceptable for most submillimeter applications; for example, the worst-case return loss due to misalignment in the WR-1.0 band (750-1100 GHz) would be 7 dB.

A flange design by Lau and Denning [4] improved the alignment between waveguides by using a male flange with a boss and a mating female flange with a close fitting receptacle.  However, this has the disadvantages of not being compatible with the older UG-387 design, not being sexless, and not having lappable contact surfaces.

Like the Lau-Denning design, the ring-centered flange uses a precisely machined boss centered on the waveguide aperture, but alignment is achieved by a ring which centers the bosses of two mating flanges. The flange is compatible with the UG-387 type, allowing components with the new flange to be connected directly to older components without adapters.  Like the UG-387 standard, it is sexless and the contact surfaces are not recessed, which facilitates cleaning and lapping to repair damaged flanges.  It also has an anti-cocking rim [5], which has been incorporated in many of the newer implementations of the UG-387 standard.  The maximum misalignment between waveguides using the new flange is 0.0012", corresponding to a worst-case return loss of 13 dB in the WR-1.0 band (750-1100 GHz).

[1] A. R. Kerr, E. Wollack, and N. Horner, "Waveguide Flanges for ALMA Instrumentation,"  ALMA Memorandum 278, 9 Nov. 1999.  http://www.alma.nrao.edu/memos/

[2] A. R. Kerr, L. Kozul and A. A. Marshall, "Recommendations for Flat and Anti-Cocking Waveguide Flanges,"  ALMA Memo 444, National Radio Astronomy Observatory, Charlottesville VA 22903, 6 Jan 2003.  http://www.alma.nrao.edu/memos/

[3] J. L. Hesler, A. R. Kerr, W. Grammer, and E. Wollack, "Recommendations for Waveguide Interfaces to 1 THz," Proc. 18th International Symposium on Space Terahertz Technology, Pasadena, 21-23 March 2007, pp.100-103.

[4] Y. Lau and A. Denning, “An Innovative Waveguide Interface for Millimeter Wave and Submillimeter Wave Applications,” 69th ARFTG Microwave Measurements Conf. Dig., Honolulu, HI, June 2007.

[5] See, e.g., http://www.flann.com/Sales/FMI_Catalogue.pdf, p. 119.