Fast Fourier Transform Spectrometer

B. Klein*, I. Krämer, S. Hochgürtel, R. Güsten, A. Bell, K. Meyer, V. Chetik

Max-Planck-Institut für Radioastronomie, Bonn, Germany

* Contact:  bklein@mpifr-bonn.mpg.de, phone +49-228-525 286

Abstract—We review the development of our digital broadband Fast Fourier Transform Spectrometers (FFTS). In just a few years, FFTS back-ends - optimized for a wide range of radio astronomical applications - have become a new standard for heterodyne receivers, particularly in the mm and sub-mm wavelength range. They offer high instantaneous bandwidths with many thousands spectral channels on a small electronic board (100 x 160 mm). Our FFT spectrometer make use of the latest versions of GHz analog-to-digital converters (ADC) and the most complex field programmable gate array (FPGA) chips commercially available today. These state-of-the-art chips have made possible to build digital spectrometers with instantaneous bandwidths up to 1.8 GHz and 8192 spectral channels.

Our latest development step is a prototype compact FFTS board with a 5 GHz / 8-bit ADC and an outstanding instantaneous bandwidth of 2.5 GHz. The board has been successfully field-tested at the APEX telescope and we present a first spectrum observed with APEX in November 2008. Finally, we provide an outlook to our on-going developments.