Ultra Wideband Cryogenic Phase Locking Loop System for Flux-Flow Oscillator

A.V. Khudchenko1,2, V.P. Koshelets1,2, P.N. Dmitriev1, A.B. Ermakov1,2, O.M. Pylypenko3

1 Institute of Radio Engineering and Electronics, Moscow, Russia

2 SRON Netherlands Institute for Space Research, Groningen, the Netherlands

3 State Research Center of Superconducting Electronics "Iceberg", Kyiv, Ukraine

* Contact: Khudchenko@hitech.cplire.ru, phone +7-495-629 3418

The work was supported by the projects: RFBR 06-02-17206, ISTC 3174, NATO SfP 981415, and Grant for Leading Scientific School 5408.2008.2

Abstract—We present an ultra wideband Cryogenic Phase Locking Loop system (CPLL). The CPLL was developed for phase locking of a Flux-Flow Oscillator (FFO) in a Superconducting Integrated Receiver (SIR), but can be used for any cryogenic terahertz oscillator. The key element of the CPLL is a cryogenic phase detector (CPD) based on superconductor – insulator – superconductor (SIS) tunnel junction. All the loop elements are placed inside a cryostat at 4.2 K; the novel CPLL system has a loop length 50 cm and the loop group delay is about 4.5 ns. Such small delay results in a CPLL synchronization bandwidth as much as 40 MHz and allows to phase-lock more than 60% of the emitted by FFO power even for FFO linewidth about 11 MHz. This fraction of phase-locked power is three times more than can be achieved with conventional room-temperature PLL. Details of the phase noise measurements for the FFO phase-locked by the CPLL will be presented.