Shot-Noise Characteristics of NbN Superconducting Tunnel Junctions

Jing Li1*, Xue-Song Cao1, Masanori Takeda2, Hiroshi Matsuo3, Zhen Wang2, and Sheng-Cai Shi1

1 Purple Mountain Observatory, CAS, Nanjing, JiangSu 210008, China

2 Kobe Advanced ICT Research Center, NiCT, Kobe, Hyogo 651-2492, Japan

3 National Astronomical Observatory of Japan, NINS, Mitaka, Tokyo 181-8588, Japan

* Contact:  lijing@mail.pmo.ac.cn, phone +86-25-8333-2229

Abstract—An 0.5-THz superconductor-insulator-superconductor (SIS) mixer incorporating with all-NbN superconducting tunnel junctions has been developed and put into real astronomical applications [1]. Even with a measured receiver noise temperature down as low as five times the quantum limit (5h/kB), this NbN SIS mixer doesn’t demonstrate reasonably good direct-detection behavior as conventional Nb SIS mixers. It is thought that the enhanced shot-noise due to the MAR (multiple Andreev reflection) effect [2] is considerably larger in direct detection than in heterodyne mixing. To better understand this difference, in this paper we mainly investigate the shot-noise characteristics of NbN superconducting tunnel junctions. Firstly, the power spectral density of the shot noise resulting from 0.5-THz NbN twin SIS junctions is measured with respect to the dc bias voltage by a low-frequency (~100 Hz) and high-frequency (a few GHz) method. The measured results are compared with simulated ones to understand how the shot noise is enhanced by the MAR effect. Secondly, the direct-detection and heterodyne mixing characteristics of the 0.5-THz NbN twin SIS junctions are measured and compared.  Detailed experimental and simulation results will be presented.

- Jing Li, Masanori Takeda, Zhen Wang, Sheng-Cai Shi, and Ji Yang, Appl. Phys. Lett., vol. 92, 222504, June 2008.

- P. Dieleman, H. G. Bukkems, T. M. Klapwijk, M. Schicke, and K. H. Gundlach, Phys. Rev. Lett., vol. 79, pp. 3486-3489, Nov. 1997.